# KNOWLEDGE INSTITUTE OF TECHNOLOGY

# (An Autonomous Institution)

Department of Mechanical Engineering

TEAM NAME: SPARKERS

CAE & CALCULATION REPORT

Calculation Report: -

1.Centre of gravity

Where,

C1- Centre of mass of battery = 2kg

C2- Centre of mass of seat = 50kg

C3- Centre of mass of shaft = 10kg

C4- Centre of mass of engine =30kg

C5- Centre of mass of chassis =25kg

C6- Centre of mass of steering = 4kg

Consider intersection of centre lines as origin,

X=  C 1x1+C2x2+C3x3+C4x4+C5x5+C6x6

𝐶1+𝐶2+𝐶3+𝐶4+𝐶5+𝐶6

2×(−10)+10×(−2.25)+50×(−2.25)+30×12.25

2+10+50+4+30+25

=1.75

Y= C1y1+C2y2+C3y3+C4y4+C5y5+C6y6

𝐶1+𝐶2+𝐶3+𝐶4+𝐶5+𝐶6

2× (−13.5) +10 × (−21.5) +50× (−5.4) +4×13+30× (−4.5)

=

2+10+50+4+30+25

= -5.0

In X direction- 19.32 inches from left side

In Y direction- 26.62 inches from front side

Finite element analysis: -

Analysis is done with ANSYS software. By using ANSYS, we get results of Front impact, side impact, rear impact and torsional.

1)Front impact:

Assuming, 2G Force

F = M × g ×N

=   170× 9.81 × 2 F

= 3335N

2)Rear impact:

Assuming, 2G Force

F = M × g ×N

= 170× 9.81 ×2F

= 3335N

3) Side impact:

Assuming, 2G Force

F = M × g ×N

= 170× 9.81 × 2 F

= 3335N

# Steering Calculation: -

| Parameters | Values |
| --- | --- |
| Track Width (TW) | 1016 mm |
| Wheelbase (WB) | 1041 mm |
| Total Weight (W) | 170 kg |
| Turning Radius (R) | 2.7m |
| Acceleration due to gravity (g) | 9.81 m/s2 |
| Coefficient of Friction static (µ) | 0.8 |
| Coefficient of Friction Dynamic (µ) | 0.017 |
| Velocity of Vehicle(v) | 8.33 m/s |

Calculations:

- Ackerman angle (α)

= tan -1 (0.5×TW / WB)

= tan -1 (0.5×1016/1041)

= 260

- Inner angle (a)

= tan-1 {WB / (R - TW /2)}

= tan-1 {1041/ (2000 -1016 /2)}

= 340

- Outer angle (b): -

= tan-1 {WB / (R + TW / 2)}

= tan-1 {1041/ (2000 + 1016/ 2)}

= 22.580

4)Actual Turning radius (R) P = (a + b) / 2

= 28.290

R = (TW / 2) + [WB / Sin P]

= (1016 / 2) + (1041/ Sin 28.29)

= 2.708 m

Analytical calculations are nearly equal to results from the Ackermann geometry.

Moment ratio

=Radius Of steering wheel / length of tripod

=6/4

=1.2

At Static Condition: -

1.Vertical force on front wheel (Fv)

= mg

= 31× 9.81

= 304.11N

2.Force on steering arm due to normal force (Fn)

=µ ×Fv

=0.8 × 304.11

#### =243.288 N

3.Steering Effort (Fst)

=Fn / (2 × moment ratio)

= 243.288/ (2×1.2)

= 101.37 N

### At Dynamic Condition: -

1.Lateral force (Fy)

= m × v² /R

=31×8.33² / 2.7

=796.6N

2.Force due to normal load (Fn)

= µ ×Fv

= 0.017 × 304.11

= 5.165 N

3.Total force on steering arm(F)

= Fn +[Ml)/ L]

= 5.165 + [17832.83 / 127]

= 145.58N

4.Steering effort (Fst)

‘= F / (2 × Moment ratio)

= 145.48 / (2×1.2)

= 60.61 N

Brake Calculation: -

Brake type=Apache 160 RTR

Diameter of rear disc=200mm

Number of pistons=2

Diameter of piston=28.5mm

Diameter of cylinder=15.87mm

Area of master cylinder=π/4d2

=π/4(15.87)/2

=197.80mm2.

Diameter of caliper=40mm

Area of caliper=π/4d2

=π/4(40)2

=1256.6mm2.

Area of piston=π/4d2

=π/4(28.5)2

=637.93* 2

=1257.87mm2

Pedal ratio=6:1

Coefficient of friction between tyre and road(𝝁₁) = 0.7

Coefficient of friction between Brake pad &Disc rotor(𝜇2) =0.4

C=Weightdistributiononfrontaxleis35%

C= Weight distribution on rear axle is 65%

Calculation: -

### 1.Normal force at front and rear axle:

NFront =mass*g*c

=170*9.81*0.35

=583.695N

NRear =mass*g*c

=170*9.81*0.65

=1084.005N

### 2.Friction force at front and rear axle;

FFront=µ1*N front

=0.7*583.695

=408.5N

FRear=µ1*NRear

=0.7*1084.005

=758.8N

### 3.Acceleration of the kart

Total Friction force= (Front +Rear)

=408.5+758.8

=1167.376 N

Acceleration of the kart

According to Newton’s second law of motion F=ma

a=𝐹/𝑚

a=1167.37/170

a=6.9ms-2

### 4.Stopping time

Kart starts from rest.

Final velocity at 10Km/hr,

v=2.7m/s

t=(v-u)/a

= (2.7-0)/6.9

t=0.4sec

Final velocity at 20Km/h,

v=5.55m/s

t=(v-u)/a

= (5.55-0)/6.9

t=0.8sec

Final velocity at 30Km/hr,

v=8.33m/s

t=(v-u)/a

= (8.33-0)/6.9

t=1.20sec

Final velocity at 40Km/hr,

v=11.11m/s

t=(v-u)/a

= (11.11-0)/6.9

t=1.6sec

Initial velocity, u=o

Final velocity at 50 Km/hr ,

v=13.8 m/s

According to newton ’first law of motion

Final velocity=initial velocity + acceleration

*Time

v=u+at t

=(v-u)/a

= (13.8-0)/6.9

t=2.0sec

Initial velocity, u=o

Final velocity at 60Km/hr,

v=16.66m/s

t=(v-u)/a

= (16.66-0)/6.9

t=2.41sec

Final velocity at 70Km/hr,

v=19.44m/s

t=(v-u)/a

= (19.44-0)/6.9

t=2.8sec

Final velocity at 80Km/hr,

v=22.22m/s

t=(v-u)/a

= (22.22-0)/6.9

t=3.2sec

### 5.Stopping distance

According to newton third law of motion 2a*d=v2-u2

d =v2/2a

Final velocity at 10Km/hr,

v=2.7m/s

d = v2/2a

d= (2.7)2/(2*6.9)

d=0.53 m

Final velocity at 20Km/hr,

v=5.55m/s

d = v2/2a

d= (5.55)2/(2*6.9)

d=2.23 m

Final velocity at 30Km/hr,

v=8.33m/s

d = v2/2a

d= (8.33)2/(2*6.9)

d=5.102m

Final velocity at 40Km/hr,

v=11.11m/s

d = v2/2a

d= (11.11)2/(2*6.9)

d=9.07m

Final velocity at 50Km/hr,

v=13.8m/s

d = v2/2a

d= (13.8)2/(2*6.9)

d=13.8 m

Final velocity at 60Km/hr,

v=16.66m/s

d = v2/2a

d= (16.66)2/(2*6.9)

d=20.40 m

Final velocity at 70Km/hr,

v=19.44m/s

d = v2/2a

d= (19.44)2/(2*6.9)

d=27.78m

Final velocity at 80Km/hr,

v=22.22m/s

d = v2/2a

d= (22.22)2/(2*6.9)

d=36.3m

### 6.Brake fluid pressure

Pedal ratio=6:1

Force on master cylinder=2500N

Force=pressure*area of the master cylinder. F=P*a

P=F/a

=2500/197.80

P=12.69N/mm2

### 7.Clamping Force

=fluid pressure*area of caliper

=12.69*1256.6

=15946.254N

#### 8.Friction on rear tyre

Force on rear=µ2* clamping force

=0.4* 15946.254

Fr=6378.50N

#### 9.Braking torque

T=Fr*L

Where,

L=Static laden radius=134.11mm

=6378.50*134.11

Torque=855.42 N m

Transmission Calculation: -

SPECIFICATION:

Engine Type: SI series

EngineDisplacement:149.5cc EngineDescription:149.5cc,4stroke

Maximum Power: 13.8 Bhp @ 8000 rpm MaximumTorque:13.4Nm@6000rpm Bore x Stroke: 57.0 x 56.4mm

Fuel Type: Petrol

No. of Cylinders:1

No. of Gears: 5

MaximumRPM:8000rpm

Shaft diameter: 30mm

Shaft length: 32”

CHAIN DRIVE SYSTEM:

Gear Box Calculation

First Gear Ratio (G1) = 2.92

Second Gear Ratio (G2) = 1.88

Third Gear Ratio (G3) = 1.38

Fourth Gear Ratio (G4) =1.08

Fifth Gear Ratio (G5) = 0.92

Primary Gear Ratio (GP)=3.47

To find chain drive ratio, we need the maximum velocity of 60 km/h.

Vmax=(Nmax*π*D*18)/( GP*Gc*G5*60*5)

Gc=Chain Drive Ratio

Nmax=Engine Maximum Speed

D = Diameter of Tyre (0.279m)

Vmax = Maximum Velocity

N1=speed of the first gear

r=radius of tyre (0.139m)

Gc = 1.733

To find the number of teeth in Driven sprocket (Ds),

Then the driver sprocket(ds)connected to the engine has 16 teeth.

CHAIN DRIVE ANALYSIS:

Driver sprocket = 16 teeth

Driven sprocket=32 teeth

Hence,

Gear ratio G=driven sprocket teeth

/Driver sprocket teeth

So

G = 32/16

=2:1

Chain pitch =0.531 inch

LetN1and N2 be the speeds of driven and driving sprockets and T1 and T2 be the number of teeth in driven and driving sprocket respectively

Then

N1/N2=T2/T1

6000 / N2 = 32/ 16

From which we obtain the driven shaft speed as N2 = 3000 rpm.